Flow Chart of the Automatic Operation

The flow chart outlines the logic and sequence of operations for the automatic control of the propeller-driven cart. Below is a textual description of the flow chart:

Code Overview:

The Arduino code controls the kinematics of the vehicle by reading the distance from the ultrasonic sensor and adjusting the motor speed accordingly. Below is a breakdown of the code:

Key Components:

- Ultrasonic Sensor:

  - Measures distance using the NewPing library.

  - Triggers and listens for the echo to calculate distance.

- Motor Control:

  - Uses PWM to control motor speed.

  - Implements braking by setting both motor pins (IN1 and IN2) to HIGH.

- Kinematics Logic:

  - Acceleration: Gradually increases speed when distance > 170 cm.

  - Deceleration: Reduces speed when distance is between 120 cm and 170 cm.

  - Stop: Activates braking when distance ≤ 120 cm.

Kinematics Calculations:

- Acceleration:

  - The cart accelerates from rest to almost max speed when the distance > 170 cm.

  - Acceleration is controlled by gradually increasing the PWM signal from 0 to 255.

d= υ0.t+12a.t2

υf= υ0+a.t

|  | 5 meters | 3 meters | 2 meters |
| --- | --- | --- | --- |
| D(m) | 2 | 1.2 | 0.3 |
| T(sec) | 4 | 3.74 | 1.8 |
| A(m/sec^2) | 0.25 | 0.23 | 0.18 |
| V(m/sec) | 1 | 0.86 | 0.324 |

- Constant Speed:

  - The cart maintains a constant speed when no obstacles are detected (distance > 170 cm).

v= dt

|  | 5 meters | 3 meters | 2 meters |
| --- | --- | --- | --- |
| D(m) | 1.3 | .3 | 0 |
| T(sec) | 1.3 | .35 | 0 |
| V(m/sec) | 1 | 0.86 | 0.324 |

- Deceleration:

  - The cart decelerates to 70% speed when the distance is between 120 cm and 170 cm.

  - Deceleration is controlled by reducing the PWM signal from 245 to 150.

υf= υ0+a.t

|  | 5 meters | 3 meters | 2 meters |
| --- | --- | --- | --- |
| D(m) | 1.5 | 1.2 | 1 |
| T(sec) | 2.34 | 2 | 1.8 |
| V final (m/sec) | 0 | 0 | 0 |
| A(m/sec^2) (-) | 0.43 | 0.43 | 0.18 |

- Stop:

  - The cart stops completely when the distance ≤ 120 cm.

  - Braking is achieved by setting both motor control pins (IN1 and IN2) to HIGH.